- Re-edited from vx5cheat.doc.  Print on A4 or legal, cutout, fold in half then fold along vertical lines for a small pocket guide. Plagiarized and edited by Art, send corrections to kc7gf@aol.com

VX-7R

Alternate Functions

These are secondary functions of the buttons. Press MONF then key,

MONI = Basic Setup: 2 SQL NFM

#1 = SCAN

#2 = Activates Dual Watch

#3 = Recall Weather memories

#4 = Activate ARTS function

#5 = Activate Smart Search

#6 = Activate Spectrum Analyzer

#7 = Activate Channel Counter

#8 = Set Menu Squelch Type:1

#9 = Enter Special Memories

#0 = Enter Set Menus

BAND = band down

MAIN & SUB=change display size

V/M = no action

NOTE: MONI/MONF buttons can be swapped.  (See misc menu item #4)

Tone Search Scanning

Set up for CTCSS or DCS.

TSQ or DCS must be displayed

Press MONF, SET, select TONE SET :2. Press BAND, press MONF, and SCAN (1). When tone flashes press BAND to lock in tone, then PTT exits. Store if needed

TX PWR Level

HI: 	5W 	Hi  (1W AM 6M)

L|||	 2.5W 	L3

L|| 	1W	L2

L|	. 05W 	L1  (.3w on 220)

Press MONF, TX PO for power level L1, L2 or L3. No icon = Hi

Supply Voltage & Current

Nominal battery:	7.4 vdc

Ext. operating:	10 – 16 vdc

Standby: (mono)	67mA (saver off)

Standby: (mono)	28mA (saver on)

Memory offset Tuning

In memory mode, press V/M.  Use DIAL to tune up/down.

Memory Scanning

Press MONF, SCAN (1) in memory mode to start scanning.

VFO Scanning

Press MONF, SCAN (1) in VFO mode to start scanning.

Microprocessor Reset (All)

Press and hold ARTS (4), BAND, and V/M while turning power on, Press MONF to reset all.

Set Mode Reset

Press and hold BAND and V/M while turning power on. Then press MONF to reinitialize all set menus.

Set Priority channel

In memory mode, press MONF 1s, select priority ch, press BAND.

Smart Search

Searches above/below current freq, stores active freqs in 31 memories. One sweep or Continuous fills all 31 memories.

 MONF, SET, select search mode scan menu #5, PTT. Select VFO in desired mono band. Press MONF, S.SCH (5). Press V/M to begin scan. To recall rotate DIAL. Exit pressing MONF, S.SCH (5).

Spectrum Analyzer

As above except use scan menu item #6 to set 1 scan or continuous, MONF, SP-ANA (6) to start/end. MAIN/SUB changes scan width.

Channel Counter

Inserts 50 dB pad in VFO mode, Set scan for +/- 5, 10, 50, 100 MHz for strongest sig. MONF, CH.CNT

Cloning

Press MONF while turning on, display reads “Clone”, Press V/M on destination radio, “clone wait”, Press BAND on source radio.

Program memory Scan

Store lower freq in L(1-20), store upper freq in matching U memory. Switch to Mem mode, select L(x) memory. Press V/M for 1s to scan.

Memory scan skip

Select Memory mode, select ch to skip. Press MONF, then SET, select Basic #13, press MAIN/SUB to select “SKIP”, PTT stores.

Preferential Scan list

Same as above except selects “PREFERENTIAL”. Musical  note will be displayed. Start by MONF, SCAN (1) from this type of chan.

# Operating Bands

## Band	Freq Range   kHz	Mode

(MHz)       	step

AM Bcst	0.5 – 1.8	9-10 	AM

SW	1.8 - 30	5-100	AM

6-M	30 - 59	5-100	FM/AM

FM Bcst  	59 -108     	5-100	FM-W

Aircraft    	108-137  	5-100    	AM

2-m         	137-174  	5-100    	FM-N

VHF TV   	174-222  	5-100    	FM-W

220 ham   	222-420  	5-100    	FM-N

Action1	225-420	5-100	FM-N

70-cm    	420-470 	5-100  	FM-N

UHF-TV   	470-800 	10-100  	FM-W

Action2     	800-999	10-100	FM-N

RPTR Uplink Freq Check

Press HM/RV Display shifts to reverse freq.  Press MONI to listen.

Masking Memories

In memory mode press MONF for 1s, turn DIAL to memory # to mask. Press V/M.  Memory is masked. Repeat to unmask.

Memory Labels

Press MONF, SET rotate DIAL to Basic menu  #11, press BAND, press MAIN or SUB or Keypad to select first character, Rotate DIAL to select second position, repeat for 8 characters, (DW (2) key selects between 7 different character sets.) Pressing PTT exits and stores.

Display must be set to small size and radio in mono band mode.

DTMF Storage

9 memories, up to 16 digits each

Press MONF, SET, TSQ/DTMF menu #8, Press BAND to enter set.

Press MAIN/SUB to select DTMF memory number

Rotate DIAL forward/back to select position, press keys to enter up to 16 #s. Press PTT to store/end

Storing memory channels

In VFO mode enter all options.  Press MONF 1 sec, select ch # with DIAL, press MONF again to store in memory

Storing “split” memories

Store RX freq as above, select TX freq. Press MONF for 1sec, within 5 sec, select same channel number as above, press and hold PTT, then press MONF to store.

Move memory to VFO

 On Memory ch, press MONF 1sec, then TX/PO (Internet button)

Send stored DTMF Num.

 Press MONF, SET, select TSQ menu #7, set to ON then press PTT to store. While pressing PTT, press numbers keys 1 – 9 to transmit stored string.

ARTS operation

Radios must be in DCS mode

Press MONF, ARTS for 1sec

Memory Group Assignment

 Select ch to assign, press MONF 1s then press # 1 to 9 to assign.

Memory Group Recall

Press MONF, SPCL(9) to recall special memories, select #2 MR Group with Dial,  Press PTT to actvate, rotate DIAL to select MG1 to MG9. Press V/M (write) to lock in MG. Set spec mem to off to exit.

Select Priority Channel

 Press MON F, 1sec then select channel to be priority, press  BAND, “P” appears next to MR.

Dual Watch Operation

 Select mem/vfo press MONF, DW to enter DW mode, V/M exits.

Set Mode Menu Summary

Press MONF, SET to enter. DIAL to select, PTT exits all menus

Basic Setup		Scan modes

1	 	SQU N-FM	1	CH COUNTER

2		SQU W-FM 	2	EDGE BEEP

3  	VFO STEP	3	RESUME

4		RX MODE	4	SCAN LAMP

5		ARS		5	SMART SEARCH

6		SHIFT		6	SPEC ANALYZER 1

7		RPT SHIFT	7	SPEC ANAL 2 AUDIO

8		MUTE SET	Measurement

9	 	KEY BEEP	1	SENSOR DISPLAY

10	LOCK MODE 	2	WAVE MONITOR

11	NAME SET	3	TEMP UNIT

12	MEM WRITE MODE	4	BARO UNIT

13	MEM SCAN MODE	5	ALTITUDE UNIT

14	HYPER WRITE	6	ALTITUDE OFFSET

Display			ARTS

1		BUSY LED	1	ARTS BEEP

2		CONTRAST	2	ARTS INTERVAL

3		DIMMER		3	CW ID PGM AND SET

4		DISPLAY MODE	5	LAMP MODE

6		LED COLOR 1	Save mode

7		LED COLOR 2	1	APO

8		METER SYMBOL	2		RX SAVE

TSQ/DCS/DTMF	3		TX SAVE

1		SQL TYPE	4		TOT

2		TONE SET	5		ON TIMER	6 OFF TIME

3		DCS SET		MISC

4	DCS COMPLIMENT	1	BCLO		11 FONT ED

5	BELL		2	HOME/REV	12 ICON SE

- 6 	SPLIT TONE	3	MON/T-CALL	13 ICON EDI

- 7	DTMF DIALER	4	MON-F MODE	14 ICON SEL

- 8	DTMF SET	5	EMERG SET	15 CLK SHF

6	HALF DEV	16 TIME SET

7	VOX SENS	17 LANGUA

8	VOX DELAY	18 ATTEN

9	BAND LINK	19 MIC MON

10	VFO MODE	20 WX ALER