Print on letter size paper (8.5x11). Page is setup so that you can print on light card DOUBLE SIDED and zig-zag fold along lines. Fits nicely into a wallet.

VX-7R

Alternate Functions

These are secondary functions of the buttons. Press MONF then key,

MONI = Basic Setup: 2 SQL NFM

#1 = SCAN

#2 = Activates Dual Watch

#3 = Recall Weather memories

#4 = Activate ARTS function

#5 = Activate Smart Search

#6 = Activate Spectrum Analyzer

#7 = Activate Channel Counter

#8 = Set Menu Squelch Type:1

#9 = Enter Special Memories

#0 = Enter Set Menus

BAND = band down

MAIN & SUB=change display size

V/M = no action

NOTE: MONI/MONF buttons can be swapped.  (See misc menu item #4)

Tone Search Scanning

Set up for CTCSS or DCS.

TSQ or DCS must be displayed

Press MONF, SET, select TONE SET :2. Press BAND, press MONF, and SCAN (1). When tone flashes press BAND to lock in tone, then PTT exits. Store if needed

TX PWR Level

HI: 5W Hi (1W AM 6M)

L|||2.5W L3

L|| 1WL2

L|.05W L1  (.3w on 220)

Press MONF, TX PO for power level L1, L2 or L3. No icon = Hi

Supply Voltage & Current

Nominal battery:7.4 vdc

Ext. operating:10 – 16 vdc

Standby: (mono)67mA (saver off)

Standby: (mono)28mA (saver on)

Memory offset Tuning

In memory mode, press V/M.  Use DIAL to tune up/down.

Memory Scanning

Press MONF, SCAN (1) in memory mode to start scanning.

VFO Scanning

Press MONF, SCAN (1) in VFO mode to start scanning.

Microprocessor Reset (All)

Press and hold ARTS (4), BAND, and V/M while turning power on, Press MONF to reset all.

Set Mode Reset

Press and hold BAND and V/M while turning power on. Then press MONF to reinitialize all set menus.

Set Priority channel

In memory mode, press MONF 1s, select priority ch, press BAND.

Smart Search

Searches above/below current freq, stores active freqs in 31 memories. One sweep or Continuous fills all 31 memories.

 MONF, SET, select search mode scan menu #5, PTT. Select VFO in desired mono band. Press MONF, S.SCH (5). Press V/M to begin scan. To recall rotate DIAL. Exit pressing MONF, S.SCH (5).

Spectrum Analyzer

As above except use scan menu item #6 to set 1 scan or continuous, MONF, SP-ANA (6) to start/end. MAIN/SUB changes scan width.

Channel Counter

Inserts 50 dB pad in VFO mode, Set scan for +/- 5, 10, 50, 100 MHz for strongest sig. MONF, CH.CNT

Cloning

Press MONF while turning on, display reads “Clone”, Press V/M on destination radio, “clone wait”, Press BAND on source radio.

Program memory Scan

Store lower freq in L(1-20), store upper freq in matching U memory. Switch to Mem mode, select L(x) memory. Press V/M for 1s to scan.

Memory scan skip

Select Memory mode, select ch to skip. Press MONF, then SET, select Basic #13, press MAIN/SUB to select “SKIP”, PTT stores.

Preferential Scan list

Same as above except selects “PREFERENTIAL”. Musical  note will be displayed. Start by MONF, SCAN (1) from this type of chan.

Set Mode Menu Summary

Press MONF, SET to enter. DIAL to select, PTT exits all menus

Basic SetupScan modes

1 SQU N-FM1CH COUNTER

2SQU W-FM 2EDGE BEEP

3  VFO STEP3RESUME

4RX MODE4SCAN LAMP

5ARS5SMART SEARCH

6SHIFT6SPEC ANALYZER 1

7RPT SHIFT7SPEC ANALYZER 2

8MUTE SETMeasurement

9 KEY BEEP1SENSOR DISPLAY

10LOCK MODE 2WAVE MONITOR

11NAME SET3TEMP UNIT

12MEM WRITE MODE4BARO UNIT

13MEM SCAN MODE5ALTITUDE UNIT

14HYPER WRITE6ALTITUDE OFFSET

DisplayARTS

1BUSY LED1ARTS BEEP

2CONTRAST2ARTS INTERVAL

3DIMMER3CW ID PGM AND SET

4DISPLAY MODE5LAMP MODE

6LED COLOR 1Save mode

7LED COLOR 21APO

8METER SYMBOL2RX SAVE

TSQ/DCS/DTMF3TX SAVE

1SQL TYPE4TOT

2TONE SET5ON TIME

3DCS SET6 OFF TIME

4DCS COMPLIMENTMISC

5BELL1BCLO11 FONT ED

6 SPLIT TONE2HOME/REV12 ICON SE

7DTMF DIALER3MON/T-CALL13 ICON EDI

8DTMF SET4MON-F MODE14 ICON SEL

5EMERG SET15 CLK SHF

6HALF DEV16 TIME SET

7VOX SENS17 LANGUA

8VOX DELAY18 ATTEN

9BAND LINK19 MIC MON

10VFO MODE20 WX ALER

ARTS operation

Radios must be in DCS mode

Press MONF, ARTS for 1sec

Memory Group Assignment

 Select ch to assign, press MONF 1s then press # 1 to 9 to assign.

Memory Group Recall

Press MONF, SPCL(9) to recall special memories, select #2 MR Group with Dial,  Press PTT to activate, rotate DIAL to select MG1 to MG9. Press V/M (write) to lock in MG. Set spec mem to off to exit.

Select Priority Channel

 Press MON F, 1sec then select channel to be priority, press  BAND, “P” appears next to MR.

Dual Watch Operation

 Select mem/vfo press MONF, DW to enter DW mode, V/M exits.

Storing memory channels

In VFO mode enter all options.  Press MONF 1 sec, select ch # with DIAL, press MONF again to store in memory

Storing “split” memories

Store RX freq as above, select TX freq. Press MONF for 1sec, within 5 sec, select same channel number as above, press and hold PTT, then press MONF to store.

Move memory to VFO

 On Memory ch, press MONF 1sec, then TX/PO (Internet button)

Send stored DTMF Num.

 Press MONF, SET, select TSQ menu #7, set to ON then press PTT to store. While pressing PTT, press numbers keys 1 – 9 to transmit stored string.

Memory Labels

Press MONF, SET rotate DIAL to Basic menu  #11, press BAND, press MAIN or SUB or Keypad to select first character, Rotate DIAL to select second position, repeat for 8 characters, (DW (2) key selects between 7 different character sets.) Pressing PTT exits and stores.

Display must be set to small size and radio in mono band mode.

DTMF Storage

9 memories, up to 16 digits each

Press MONF, SET, TSQ/DTMF menu #8, Press BAND to enter set.

Press MAIN/SUB to select DTMF memory number

Rotate DIAL forward/back to select position, press keys to enter up to 16 #s. Press PTT to store/end

Operating Bands

BandFreqkHzMode

(MHz)       step

AM Bcst0.5 – 1.89-10 AM

SW1.8 - 305-100AM

6-M30 - 595-100FM/AM

FM Bcst  59 -108     5-100FM-W

Aircraft    108-137  5-100  AM

2-m         137-174  5-100  FM-N

VHF TV   174-222  5-100  FM-W

220 ham   222-420  5-100  FM-N

Action1225-4205-100FM-N

70-cm    420-470 5-100  FM-N

UHF-TV   470-800 10-100  FM-W

Action2     800-99910-100FM-N

RPTR Uplink Freq Check

Press HM/RV Display shifts to reverse freq.  Press MONI to listen.

Masking Memories

In memory mode press MONF for 1s, turn DIAL to memory # to mask. Press V/M.  Memory is masked. Repeat to unmask.